Project Description

Title

IR-Triggered Robotic Hand with Integrated Vital Sign Monitoring (Heart Rate, SpO₂, and Temperature) via ESP32 and Adafruit IO Dashboard

1. Objective

To design and build a 3D-printed robotic forearm-wrist-hand assembly that autonomously detects the presence of a human hand, physically folds its fingers to gently secure the hand, acquires vital sign data (heart rate, SpO₂, and body temperature) through an onboard sensor, computes and displays averaged readings on a live dashboard, and then releases the hand — prioritizing user comfort and measurement accuracy throughout.

2. Functional Overview / Workflow

- Idle state: Robotic hand remains open, IR sensor on the palm continuously monitors for presence.

- Detection: When a person places their hand on the palm, the IR sensor detects contact/proximity.

- Folding: The ESP32 triggers the servo motor, which folds the fingers via a tendon-driven mechanism to gently secure the person's hand.

- Vitals acquisition: The MAX30102 sensor (positioned on the palm) continuously measures heart rate, SpO₂, and temperature over a 5–6 second window.

- Data processing: Three sample readings are captured per parameter; the average of each is computed.

- Data output: Averaged values (1 HR, 1 SpO₂, 1 Temperature) are pushed from the ESP32 to the Adafruit IO dashboard over Wi-Fi.

- Hold & release: The hand remains folded for 5 seconds after measurement completion, then automatically unfolds back to the idle/open state.

- Comfort priority: Folding speed, grip force, and timing are all tuned so the experience is safe and non-restrictive for the user.

3. Scope

This is a prototype-stage project intended for project screening/demonstration. The focus is on a working end-to-end pipeline (mechanical actuation → sensing → data processing → dashboard visualization), not on medical-grade accuracy or industrial durability.

4. Mechanical Design

- Design source: GrabCAD (reference/base CAD models for forearm, wrist, and finger geometry).

- Fabrication: 3D printed forearm, wrist, and 5-finger hand assembly.

- Actuation mechanism: Single-servo, tendon-driven underactuated hand — one servo pulls a common tendon/wire routed through channels in all 5 fingers to close them simultaneously; fingers return to open position via elastic/spring return.

- Sensor placement: IR sensor and MAX30102 both mounted on the palm, within contact area of the user's hand/fingers.

5. Hardware Requirements

| Component | Purpose | Notes |
| --- | --- | --- |
| ESP32 Dev Board (e.g., ESP32-WROOM-32 DevKit V1) | Main controller — sensor data acquisition, servo control, Wi-Fi/dashboard communication | Needs sufficient GPIO for I2C + servo PWM + IR digital input |
| MAX30102 Sensor Module | Heart rate, SpO₂, and temperature sensing | I2C interface |
| IR Sensor (reflective/proximity type) | Detects hand placement on palm | Digital or analog output, placed on palm |
| Servo Motor — MG90S / SG90 (metal-gear preferred: MG90S/MG95) | Finger folding/unfolding actuation | Choose based on required torque for tendon pull; MG90S offers more durability than SG90 |
| 3D-printed hand/wrist/forearm parts | Mechanical structure | Printed from GrabCAD-based design |
| Tendon wire/string + return springs or elastic | Underactuated finger mechanism | For 1-servo, 5-finger actuation |
| Separate 5V power supply for servo | Prevents ESP32 brownout from servo stall current | Common ground with ESP32 required |
| Jumper wires, breadboard/perfboard, connectors | Wiring and prototyping | — |
| USB cable | ESP32 programming and power (during development) | — |

6. Software Requirements / Tech Stack

| Layer | Tool / Library | Purpose |
| --- | --- | --- |
| IDE | Arduino IDE (with ESP32 board package installed) | Firmware development and upload |
| Language | C/C++ (Arduino framework) | ESP32 firmware |
| Sensor Library | SparkFun MAX3010x / MAX30102 library | HR, SpO₂, temperature reading |
| Servo Control | ESP32Servo library | PWM-based servo control on ESP32 |
| Connectivity | ESP32 WiFi library | Connects ESP32 to local network |
| Dashboard | Adafruit IO + Adafruit IO Arduino library (MQTT/HTTP) | Live display of averaged HR, SpO₂, Temperature |
| Version management | Careful tracking of library versions (Adafruit IO Arduino lib + ESP32 board package) | Precaution against dependency/compatibility issues previously encountered |

A few notes on the wiring:

- MAX30102 → ESP32: I2C bus (SDA → GPIO21, SCL → GPIO22 by default), plus 3.3V and GND.

- IR sensor → ESP32: digital output pin into GPIO4 (any free digital-capable pin works — adjust in firmware).

- ESP32 → Servo: PWM signal from GPIO13 to the servo's signal pin.

- Servo power: pulled from a separate external 5V supply, not the ESP32's onboard regulator — servo stall current can brown out the ESP32 otherwise.

- Common ground: the external supply's GND must be tied back to the ESP32's GND (dashed line) so the PWM signal reference is shared — this is easy to miss and causes erratic servo behavior if skipped.